# METHOD OF STATEMENT

Date: 05th Jan 2018

Description of work

Crane to pull out machine from container onto lorry chassis, using forklift to unload from lorry, transport to unloading area, insert rollers, shift and position to designated location

Machine Spec

Extruder JYZ 300-M Precision Preformer

Dim: 600 x 135 x 185 cm

Wt: 6, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

10/01/2018   -   10:00 am

Location of Operation

#### No 10 Tuas South St 5

CDI Multiseals Pte Ltd

Equipment Use

1 x lorry loader

Pallet truck/rollers

Hand jack/woodblock

1 x 10 ton forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, helmet, etc...)

M.O.M certified forklift operator

# Job Scope

# Lorry crane to park at designated location (back to back with container)

# Crane to pull out machine from container onto lorry chassis

# Forklift to unload machine from lorry chassis

# Transport machine to unloading area

# Insert rollers to machine base for the move

# Transport machine into workshop

# Shift and position machine to the designated location

# Unload machine from rollers and insert levelling pad under machine base

# Once job complete, team leader will instruct the team to perform proper housekeeping,

# all debris, if any will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175