# METHOD OF STATEMENT

Date: 12th May 2023

Description of work

Forklift to unload 2 crate of 1 unit machine from lorry chassis, uncrate, transport to unloading area, shift and position into production area at ground floor

Machine Spec

Datron Neo machine

Crate 1: 148 x 103 x 216 cm   /   Wt: 870 kgs

Crate 2: 143 x 128 x 147 cm    /   Wt: 318 kgs

Date of Operation

15/05/23

Time of Operation

09:45 am

Location of Operation

#### 12 Ayer Rajah Crescent

Thales Singapore

Equipment Use

1 x lorry crane

Pallet truck

1 x forklift

PVC plate (to lay out for tiled flooring protection during transportation of machine to designated location)

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# Deploy lorry crane at designated unloading bay

# Using forklift to fork down crate from lorry to the ground

# Uncrate machine crate and put aside

# Loosen all bolts and nuts

# Forklift to fork up and detach machine from base

# Transport and unload machine at loading bay

# Using pallet truck to transport machine into production area at ground floor

# Position machine at the designated cleanroom

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175