# METHOD OF STATEMENT

Date: 30th Mar 2017

Description of work

A) To shift out machineries (as per quotation) from customer ground floor by using pallet truck/rollers and position into customer premise at 3rd floor via cargo lift

B) To shift out 2 unit machine (as per quotation) from customer ground floor, using forklift to fork machine onto platform, hoist up to 3rd floor, shift and position to designated location

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Location of Operation

#### 16 Tampines Industrial Crescent

Jabil Circuit (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry crane

Pallet truck, rollers

1 x 4.5 ton forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent personnel who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris, if any will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, mobile/lorry crane operator and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc…safety shoe, safety helmet, safety harness)

Lifting machine and lifting gears with MOM certificate

Lifting operation are supervised by MOM certified lifting supervisor, rigger, signalman

# Job Scope

# (A)

# To move out machineries from ground floor

# Using pallet truck/rollers to shift machine to 3rd floor via cargo lift

# Position machine to the designated location

# Unload machine from pallet truck/rollers and insert levelling pad under machine bed

# (B)

# Mobile crane to park at designated location for the hoist

# Fork up and load machine onto platform

# Strap and secured machine onto platform with strapping belt

# Hoist up platform together with machine to 3rd level opening

# Stabilize platform effectively and ensure it is fully rested on the opening slab

# Use belt to secured front left and right of platform to prevent platform swing outward

# Insert pallet truck/rollers to machine base

# Shift out machine from platform and position into customer premise

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175