# METHOD OF STATEMENT

Date: 30th Apr 2018

Description of work

To hoist down 1 unit forklift using crane and using forklift to unload 2 unit machine from lorry chassis, transport to loading bay, shift and position into customer premise at ground floor by using pallet truck

Date / Time of Operation

TBC

Location of Operation

#### 73 Nanyang Drive, Valley Block

Equipment Use

2 x lorry crane

Pallet truck

1 x 2.5/3 ton forklift

Wire sling/shackle

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lorry crane to park at the designated location

# Hoist down forklift from lorry chassis

# Using forklift to fork down machine from lorry chassis

# Transport machine to unloading bay

# Unload machine at unloading bay

# Using pallet truck to shift machine into premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any, will be clear and dispose

# Remark

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175