# METHOD OF STATEMENT

Date: 29th Jan 2018

Description of work

To unload 3 cases from lorry chassis using forklift, transport cases to unloading area, uncrate, fork and detach machine from base, shift and position into designated location inside factory by using pallet truck

Machine Spec

3 crate of 1 set Xray machine

CRATE 1 Dim: 2.7 x 1.9 x 2.4 m / Wt: 1, 500 kgs

CRATE 2 Dim: 2 x 1.4 x 1.5 m

CRATE 3 Dim: 1.5 x 0.7 x 1.08 m

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

08/03/2018   -   11:00 am

Location of Operation

#### 801 Toa Payoh Lorong 7

WBL Building, O’conner’s Annex Block

Equipment Use

1 x lorry loader

1 x 3 ton Toyota battery forklift

Pallet truck

Metal plate (to lay out for machine pathway)

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet, etc...)

M.O.M certified forklift operator

# Job Scope

# Forklift to unload cases from lorry chassis

# Transport cases to unloading area

# Uncrate machine case

# Fork up and detach machine from base

# Transport and unload machine at door entrance

# Using pallet truck to transport machine into premise

# Shift and position machine to the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175