# METHOD OF STATEMENT

Date: 20th Aug 2019

Description of work

To shift out following machine from mezzanine floor, fork down using forklift, transport to outside factory and load onto lorry chassis

Machine Spec

To relocate within SDE building at ground floor

2 units Isel CNC Cutter machine

From mezzanine floor to temporary storage area at ground floor

1 unit Oven

2 unit Laser machine

1 unit Formech 1372 Vacuum Forming machine

1 unit Versatile Cutter machine

Date of Operation

21/08/2019

Time of Operation

09:45 am

Location of Operation

NUS - Blk SDE3 #01-11

Design Fabrication Workshop, Department of Architecture

Equipment Use

1 x 633 lorry loader

1 x 4.5 ton forklift

Rollers/pallet truck

Wire/webbing sling

Canvas (to cover up machine at temporary storage area)

Obstacles

No obstacles on crane loading/unloading area.

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# To shift out machine to near mezzanine floor opening

# Using forklift to fork up machine at the center of gravity

# Slowly lift up machine to check on balancing

# If machine is well balanced on the fork, slowly lower down from mezzanine floor

# Transport out to loading bay

# Load machine onto lorry chassis

# Secured machine with strapping belt

# Transport machine to next building

# Unload machine from lorry chassis

# Shift machine into premise

# Position at temporary location

# Cover up machine with canvas

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175