# METHOD OF STATEMENT

Date: 25th Aug 2025

Description of work

To fork up 1 unit machine to mazz floor using forklift, transport to outside factory, load onto lorry chassis and deliver to new location

Machine Spec

1 unit Mazak QT Compact 100

Dim: 260 x 170 x 175 cm / Wt: 4, 600 kg

1 unit Bar Feeder

1 unit Chip Conveyor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/01/25

Time of Operation

10:00 am

Location of Operation

2 Seletar Aerospace Link

Avimac

Equipment Use

2 x 633 lorry loader

1 x 10 ton forklift

Rollers/pallet truck

Metal plate/canvas (for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator will be involved in this operation

# Job Scope

# To shift out machine to near mazz floor opening

# Using forklift to fork up machine at the center of gravity

# Slowly lift up machine to check on balancing

# When machine is well balanced on the fork, reverse forklift slowly

# Lower down machine to near ground level

# Transport out to loading bay

# Load machine onto lorry chassis

# Secured machine with strapping belt prior to moving out of customer premise

# Transport and deliver machine to new location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175