# METHOD OF STATEMENT

Date: 25th Oct 2019

Description of work

To shift out 2 unit machine from 2nd floor to near loading area, fork down using forklift, transport to outside factory, load onto lorry chassis and deliver to new factory

Machine Spec

2 unit Mitsubishi Sinker EDM EA8   - To fork down from 2nd floor

1 unit Sodick ALN600G Wirecut   - From ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

05/11/2019   -   09:45 am

Location of Operation

Windsor Airmotive Asia Pte Ltd

Collection – 21 Changi North Crescent

Deliver – 38 Changi North Crescent

Equipment Use

1 x 633 lorry loader

1 x 4.5 ton forklift

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for floor protection)

Obstacles

No obstacles on crane loading/unloading area.

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# To shift out machine to near 2nd floor opening

# Using forklift to fork up machine at the center of gravity

# Slowly lift up machine to check on balancing

# When machine is well balanced on the fork, slowly lower down from 2nd floor

# Transport out to loading bay

# Load machine onto lorry chassis

# Secured machine with strapping belt

# Transport machine to new building

# Unload machine from lorry chassis

# Shift machine into premise at ground floor

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175