# METHOD OF STATEMENT

Date: 28th Jan 2021

Description of work

To unload 1 case from lorry chassis to the ground, uncrate, forklift to fork up to 2nd floor, using pallet truck to shift and position to location

Machine Spec

1 unit Equipment

Dim: 75 x 75 x 185 cm

Wt: 140 kg

Date of Operation

02/02/2021 (Tuesday)

Time of Operation

Est: 11:00 am

Location of Operation

NUS, 1 Engineering Drive 2

Block EW1, Department of Civil & Environment Engineering

Equipment Use

1 x lorry loader

1 x 4ton forklift

Pallet truck

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade off work area with safety cones, red/white tape and signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard P.P.E (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# Using forklift to unload case from lorry chassis to the ground

# Uncrate case

# Load equipment onto forklift fork

# Secured equipment firmly onto forklift fork with strapping belt

# Fork up equipment slowly to check on balancing

# If not balanced, readjust equipment till it is well balanced on the fork

# Fork up slowly to 2nd floor

# Lower down equipment and let it sit onto woodblock

# Insert pallet truck under equipment base

# Shift and transport into workshop

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175