# METHOD OF STATEMENT

Date: 25th July 2019

Description of work

1) To shift out 1 case from corridor, uncrate top cover and 4 side of case, using forklift to fork up onto lorry chassis, transport from workshop W2 to W216, forklift to unload from lorry chassis, fork up and unload into corridor at 2nd floor, shift and position to location together with the base

2) Supply and assemble ‘A’ frame structure with chain block to lift up gear box for your engineer to install gear box stand

Equipment Spec

1 unit Gear Box

Dim: 170 x 168 x 205 cm

Wt: 4, 000 kgs

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

500 Dover Rd, Singapore Polytechnic

From W2 to W216

Equipment Use

1 x lorry crane

1 x 4.5 ton forklift

Rollers/pallet truck

‘A’ frame structure

1 x 5 ton chain block

Obstacles

To clear all obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in the operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator

# Job Scope

# Deploy lorry crane at the designated location

# Hoist down forklift from lorry chassis

# Shift out 1 case from corridor to loading area

# Uncrate top cover and 4 side of case

# Fork up base and load onto lorry chassis

# Secured gear box with strapping belt

# Transport base from W2 to W216

# Unload base from lorry chassis

# Fork up and unload base together with gear box onto corridor

# Shift and position into workshop

# Using ‘A’ frame structure with 5 ton chain block to lift up gear box to 5ft high for engineer to install gear box stand

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175