# METHOD OF STATEMENT

Date: 15th Aug 2018

Description of work

To unload 1 case from lorry chassis, uncrate, using forklift to detach machine from base, transport into inside factory, fork up to 2nd floor, using pallet truck to shift and position into CMM room

Machine Spec

1 unit New Global S Green 7.10.7 CMM

Dim: 1277 x 1905 x 2777 mm

Wt: 1, 255 kgs

Date / Time of Operation

23/08/2018   -   09:30 am

Location of Operation

6 Seletar Link

GE Aviation Engine Services

Equipment Use

2 x lorry crane

1 x 4.5 ton forklift

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine position

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified forklift operator

# Job Scope

# Lorry crane to park at the designated location

# Hoist down forklift from lorry chassis

# Using forklift to fork down machine case

# Uncrate case

# Fork up and detach machine from base

# Secured machine firmly with strapping belt

# Transport machine into inside factory

# Fork up and unload machine at 2nd floor

# Using pallet truck to shift and position machine into tool room

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

# Remark

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175