# METHOD OF STATEMENT

Date: 28th Sep 2023

Description of work

To unload 2 case from lorry chassis to the ground, uncrate, transport and position 1 unit at ground floor and another 1 unit using forklift to fork up to mezzanine floor

Machine Spec

2 unit 7.10.7 CMM machine

Case Dim: 1908 x 1300 x 1900 mm each

Wt: 1, 300 kg each

Date of Operation

Estimate:

09/10/23

Time of Operation

09:40 am

Location of Operation

21 Changi North Crescent

Windsor Airmotive Asia Pte Ltd

Equipment Use

2 x lorry loader

1 x 4.5ton forklift

Pallet truck

Wire/webbing sling

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Lorry crane to hoist down forklift from lorry chassis to the ground

# Using forklift to unload case from lorry chassis

# Unload case to the ground

# Uncrate case and put aside

# Forklift to fork up and detach machine from base

# Transport to door entrance

# Using pallet truck to shift and position at ground floor

# Using forklift to transport another unit to near mezzanine floor

# Secured machine firmly onto forklift fork with strapping belt

# Fork up machine and unload at mezzanine floor

# Using pallet truck to shift and position into CMM room

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175