# METHOD OF STATEMENT

Date: 13th Mar 2018

Description of work

To unload 11 cases from 1 x 20ft FR, 1 x 40ft HQ and 1 x 20ft GP container, uncrate, load onto platform, mobile crane to hoist up to 3rd floor, shift and position to designated location

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

15/03/2018   -   09:45 am

Location of Operation

#### No 70 Tuas South St 5

Tecnisco Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Pallet truck, rollers

1 x 4.5 ton forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent personnel who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger, signalman

# Job Scope

# Mobile/lorry crane to park at designated location for the hoist

# Unload machine from container to the ground

# Uncrate machine case

# Load machine onto platform

# Secured machine onto platform with strapping belt

# Hoist up platform together with machine to 3rd level opening

# Stabilize platform effectively and ensure it is fully rested on the opening slab

# Use belt to secured front left and right of platform to prevent platform swing outward

# Insert pallet truck/rollers to machine base

# Shift out machine from platform and position to the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor/lifting supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175