# METHOD OF STATEMENT

Date: 06th Feb 2018

Description of work

To unload 4 cases from forwarder lorry, uncrate, forklift to fork furnace onto platform, mobile crane to hoist up to 3rd floor, shift and position to designated location

Machine Spec

Case 1: 295 x 197 x 300 cm / NW: 2, 270 kgs

Case 2: 300 x 115 x 255 cm / NW: 780 kgs

Case 3: 135 x 130 x 185 cm / NW: 790 kgs

Case 4: 305 x 100 x 85 cm / NW: 170 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

07/02/2018   -   Afternoon slot

Location of Operation

#### No 70 Tuas South St 5

Tecnisco Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

Pallet truck, rollers

1 x 4.5 ton forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent personnel who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry crane operator and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc.., safety shoe, safety helmet, safety harness)

Lifting machine and lifting gears with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger, signalman

# Job Scope

# Mobile crane to park at designated location for the hoist

# Shift out machine to loading area

# Uncrate machine case

# Using forklift to fork up and detach machine from base

# Load machine onto platform

# Secured machine onto platform with strapping belt

# Hoist up platform together with machine to 3rd level opening

# Stabilize platform effectively and ensure it is fully rested on the opening slab

# Use belt to secured front left and right of platform to prevent platform swing outward

# Insert pallet truck/rollers to machine base

# Shift out machine from platform and position to designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor/lifting supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175