# METHOD OF STATEMENT

Date: 10th July 2018

Description of work

To unload 10 cases of machine and accessories from 1 x 40ft FR and 1 x 40ft GP container, uncrate, detach from base using forklift, transport to main entrance, insert rollers, shift and position into

Machine Spec

1 unit MAGIC 2.2 Machine

Machine Case: 427 x 263 x 381 cm

Wt: 12, 072 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

20/07/2018   -   09:45 am

Location of Operation

#### NTU - 50 Nanyang Ave

Equipment Use

1 x 50 ton lorry crane

1 x low bed

1 x 10 ton forklift

1 x 4.5 ton forklift

Rollers/pallet truck

Wire/webbing sling

Canvas/metal sheet (to protect rubber tile flooring)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade work area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gears with M.O.M certificate

Operation are supervised by M.O.M certified lifting supervisor, rigger, signalman and forklift operator

# Job Scope

# To unload 10 cases from container using lorry crane and forklift

# Uncrate machine case

# Using 2 x forklift to fork up main machine and detach from base

# Shift machine base away

# Lower down machine slowly

# Insert rollers to machine base

# Transport machine into workshop

# Shift and position machine to the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Remark:

# MOS subject to change by supervisor/lifting supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175