# METHOD OF STATEMENT

Date: 26th July 2019

Description of work

To hoist up 2 cases of Gear Box from corridor onto lorry chassis, transfer to another location, unload from lorry chassis, uncrate, using forklift to detach from base, transport and unload onto school corridor, to shift and position into workshop using rollers/pallet truck

Gear Box Spec

Dim: 170 x 168 x 205 cm

Wt: 4, 000 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

500 Dover Rd, Singapore Polytechnic

From W2 – W216

Equipment Use

1 x 633 lorry loader

1 set wire/webbing sling

Rollers, pallet truck

1 x 4.5 ton forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent unauthorized persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

# Job Scope

# Deploy lorry crane at the designated location

# Hoist up cases from corridor onto lorry chassis

# Transfer to next location

# Unload case from lorry chassis to the ground

# Uncrate case

# Using forklift to fork up and detach gear box from base

# Transport and unload gear box on the corridor

# Using rollers/pallet truck to shift gear box into workshop

# Position gear box at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175