# METHOD OF STATEMENT

Date: 07th June 2021

Description of work

To unload 1 set machine from lorry chassis, using pallet truck to shift into workshop at ground floor

Machine Spec

1 set Assembly machine

Main: 180 x 105 x 70 cm

Loader: 130 x 85 x 180 cm

Conveyor: 180 x 20 x 100 cm

Control Panel: 100 x 20 x 100 cm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/06/21

Time of Operation

Est: 11:30 am

Location of Operation

601 Rifle Range Rd

ST Kinetic

Equipment Use

1 x 633 lorry loader

1 set wire/webbing sling

Pallet truck

1 x manual lifter

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator and will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Hoist down machine from lorry chassis to the ground

# To tilt down machine

# Using pallet truck to transport machine into workshop

# Tilt back machine inside workshop room

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175