# METHOD OF STATEMENT

Date: 15th Apr 2024

Description of work

To hoist down 1 unit machine from lorry chassis, using rollers/pallet truck to shift and position into premise

Machine Spec

1 unit Hot Rolling Milling machine

Wt: 3, 590 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/04/24

Time of Operation

09:45 am

Location of Operation

5 Cleantech Loop,

Cleantech Two, #01-01 CT2B

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

Rollers/pallet truck

Woodblock

Canvas, metal plate (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated unloading area

# Uncrate machine case

# Attached sling to crane hook and secured to rigging point of machine

# Hoist up machine from lorry chassis

# Swing crane and lower down machine to near ground level

# Lay out canvas, metal plate onto floor for protection

# Using rollers/pallet truck to transport machine to 3rd floor via cargo lift

# Position machine at the designated location

# Unload machine from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175