# METHOD OF STATEMENT

Date: 10th Oct 2025

Description of work

1) To hoist down 3 cases of 1 unit machine from lorry chassis to the ground, uncrate, shift and position into workshop at ground floor

2) To shift out 1 unit old machine from workshop to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

New machine

MPI machine

Case 1: 366 x 104 x 153 cm / Wt: 828 kg

Case 2: 122 x 122 x 71 cm

Case 3: 91 x 91 x 101 cm

Total wt: 2, 131 kg

Old machine

Magnaflux machine

Dim: 400 x 110 x 185 cm

Wt:

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/10/25

Time of Operation

10:00 am

Location of Operation

505A Airport Rd

ST Engineering Aerospace Ltd

Equipment Use

1 x 633 lorry loader

Webbing sling

Pallet truck

Crowbar/woodblock

Canvas, PVC/plywood (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

# To hoist down cases from lorry chassis to the ground

# Uncrate case and put aside

# Riggers to insert sling to crane hook

# Secured sling to rigging point of machine

# Lorry crane to hoist up and detach machine from base

# Swing machine away from base and lower down to near ground level

# Using pallet truck to transport machine into workshop

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Shift out old machine to loading area

# Hoist up onto lorry chassis and transport out to scrapyard for disposal

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175