# METHOD OF STATEMENT

Date: 08th Dec 2025

Description of work

To hoist down 3 cases of 1 unit machine from lorry chassis to the ground, uncrate, shift and position into production at ground floor

Machine Spec

3 cases of 1 unit LH67 CMM

Case 1: 210 x 160 x 200 cm /

N/G Wt: 1, 300 kg / 1, 350 kg

Case 2: 210 x 150 x 110 cm

N/G Wt: 360 kg / 400 kg

Case 3: 130 x 95 x 122 cm

N/G Wt: 135 kg / 150 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/12/25

Time of Operation

10:00 am

Location of Operation

9 Seletar Aerospace Link

Hamilton Sundstrand Pacific Aerospace Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Webbing sling

Pallet truck

Crowbar/woodblock

Canvas, PVC sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at designated unloading area

# Set up crane with outrigger fully extended and resting on timber mats as base plate

# To hoist down cases from lorry chassis to the ground

# Uncrate case and put aside

# Riggers to insert sling to lorry crane hook and secured to rigging point of machine

# Lorry crane to hoist up and detach machine from base

# Load up machine onto machine stand

# Using pallet truck to transport machine into production

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175