# METHOD OF STATEMENT

Date: 18th Nov 2019

Description of work

To unload 4 cases of 1 unit machine from lorry chassis, uncrate, hoist up and detach machine from base, using pallet truck to shift and position to location at ground floor

Machine Spec

HP 10.7.6 CMM Machine

Box 1: 220 x 150 x 223 cm / Wt: 2, 150 kgs

Box 2: 104 x 104 x 152 cm / Wt: 250 kgs

Box 3: 120 x 80 x 172 cm / Wt: 162 kg

Box 4: 120 x 80 x 172 cm / Wt: 144 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

25/11/2019   -   09:45 am

Location of Operation

3 Cleantech Loop #01-01

Advanced Remanufacturing & Technology Centre (ARTC)

Equipment Use

2 x 633 lorry loader

Wire/webbing sling

Pallet truck

Canvas, plywood, metal plate (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated location

# Hoist down case from lorry chassis to the ground

# Uncrate case

# Hoist up and detach machine from base

# Unload machine at door entrance

# Lay out canvas, metal plate onto floor for protection

# Using pallet truck to transport machine into production

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175