# METHOD OF STATEMENT

Date: 20th July 2020

Description of Work

Transport lorry crane into plant, using crane and forklift as support to transport into plant, upright and position tank to location

Tanks Spec

6 unit FRP Tank

340 x 550 cm

Wt:1, 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

20-24/07/20

Time of Operation

09:45 am

Location of Operation

92 Tuas South Ave 3

PUB

Equipment Use

1 x 633 lorry crane – PALFINGER PK100002E

Wire sling

4 pcs shackle (SWL: 8.5 ton)

1 x 4. ton forklift

1 x boom lift

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location inside plant

- Insert sling onto lifting point of tank

- Lorry crane to hoist up and upright tank

- Insert sling at bottom of tank and attached to 4.5ton forklift

- Using 4.5ton forklift to support when lorry crane is hoisting to prevent tank from swinging

- Position tank at the designated location

- Riggers to release sling from tank by using boom/scissor lift

- Repeat process till all tanks are position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175