# METHOD OF STATEMENT

Date: 09th June 2021

Description of work

To shift out and hoist down 1 unit machine from 2nd floor through swing gate opening by using lorry crane

Machine Spec

1 unit CG machine

Dim: 270 x 140 140 cm

Wt: 3, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

26/06/21 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### Blk 8 Lorong Bakar Batu #02-10

Feimus Engineering Pte Ltd

Equipment Use

1 x 633 PALFINGER lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Shift out machine from production to near swing gate opening

- Insert one end of sling to machine hoisting point and another end onto crane hook

- Hoist up 1ft above ground to check on balancing

- When well balanced, hoist down machine slowly to the ground

- Load machine onto lorry chassis

- Secure machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175