# METHOD OF STATEMENT

Date: 26th Sep 2025

Description of work

1) To hoist down 2 units machine from 5th floor through shutter door opening by using platform & mobile crane

2) To hoist up 2 units new machine to 5th floor through shutter door opening by using platform & mobile crane

Machine Spec

To hoist down old machine

1 unit GX1000 CNC

Dim: 2946 x 2133 x 2717 mm / Wt: 5, 000 kg

1 unit VMC600

Dim: 2200 x 1900 x 2500 mm / Wt: 5, 000 kg

To hoist up new machine

1 unit Micro Dynamics Mega 30V

Dim: 2750 x 2300 x 2550 mm / Wt: 4, 890 kg

1 unit Brother W1000Xd2

Dim: 2410 x 2233 x 2633 mm / Wt: 3, 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/11/25 (Saturday)

Time of Operation

10:00 am

Duration of Operation

Estimate:

10:00 am – 18:00 pm

Location of Operation

#### 10 Tampines North Dr #05-04

#### Distinct Engineering & Trading Pte Ltd

Equipment Use

2 x 50ton mobile crane

2-3 x lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

Job Scope

- Deployment of 2 x mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- 1 x mobile crane to hoist up platform to 5th floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load onto platform

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist down machine from platform to the ground

- Load up machine onto forwarder lorry

- To hoist up new machine to 5th floor and load onto platform

- Shift out from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175