# METHOD OF STATEMENT

Date: 26th Sep 2025

Description of work

1) To hoist down 3 units bar feeder from 5th floor through shutter door opening by using platform & mobile crane

2) To hoist up 2 units Bar Feeder to 6th floor through shutter door opening by using platform & mobile crane

Bar Feeder Spec

400 x 60 x 120 cm

Wt: 300 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

07/10/25 (Tuesday)

Time of Operation

10:00 am

Duration of Operation

Estimate:

10:00 am – 14:00 pm

Location of Operation

#### 10 Tampines North Dr #06-08

#### Turntech Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1-2 x lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

Job Scope

- Deployment of mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Load bar feeder onto platform

- Secured bar feeder onto platform with strapping

- Mobile crane to hoist up platform together with bar feeder to 6th floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers onto bar feeder stand

- Shift out from platform

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175