# METHOD OF STATEMENT

Date: 20th Nov 2025

Description of work

To hoist down 1 unit machine from 5th floor through shutter door opening by using platform and mobile crane and load up onto customer arrange forwarder lorry

Machine Spec

1 unit used CNC machine

220 x 200 x 220 cm

Wt: 7, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

02/12/25

Time of Operation

09:00 am

Location of Operation

18 Woodlands Loop

Edmund Optics (S) Pte Ltd

Equipment Use

1 x 100ton mobile crane

1 x 633 lorry crane

1 x working platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- 1 x mobile crane to hoist up platform to 5th floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and load into platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist down platform together with machine to the ground

- Load up machine onto forwarder lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175