# METHOD OF STATEMENT

Date: 11th Oct 2024

Description of work

Hoisting of machineries at 2nd & 3rd floor through shutter gate opening by using platform and mobile/lorry crane

Machine Spec

Hoist down from 3rd floor

2 unit used machine (to scrapyard for dispose)

Wt:  kg

Hoist down from 2nd floor

1 unit Hi Tech 230 / Wt: 6, 800 kg (from 2nd floor to ground floor)

1 unit R650 / Wt: 3, 800 kg (load up onto Malaysia lorry)

2 unit TC31 / 2, 200 kg each (load up onto Malaysia lorry)

Hoist up to 3rd floor

1 unit Conventional Grinder

Wt: 3, 600 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/10/24 (Wed)

Time of Operation

09:45 am

Location of Operation

#### 5 Buruh St

#### Globaltronic Precision Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry crane

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Mobile crane to hoist up platform to 3rd floor shutter gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- To shift out machine from production into platform

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist up machine from platform

- Swing crane and lower down machine to the ground

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175