# METHOD OF STATEMENT

Date: 11th Aug 2025

Description of work

To hoist down machineries from 2nd floor through wall hacking opening by using platform and mobile/lorry crane, load up onto lorry and deliver to Eric Wong warehouse

Machine Spec

2 units Brother TC-S2D / Dim: 160 x 215 x 245 cm / Wt: 2, 400 kg each

1 unit Feeler QM-40SA / Dim: 280 x 220 x 245 cm / Wt: 6, 300 kg

1 unit Lathe machine / Dim: 220 x 110 x 150 cm / Wt: 2, 200 kg

2 units 5HP Milling / Dim: 180 x 180 x 200 cm / Wt: 2, 000 kg each

1 unit Jaw Saw

1 unit Table Top CMM

1 unit Granite

2 units Compressor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/08/25 (Sat)

Time of Operation

09:45 am

Location of Operation

#### Blk 1004 Toa Payoh Ind Pk #02-1479

Estech Engineering (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

4 x 633 lorry crane

1 x platform

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Mobile crane to hoist up platform to 2nd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Shift out machine from production and load into platform

- Riggers to attached sling to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist up machine from platform

- Swing crane and lower down machine to the ground

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to being transport out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175