# METHOD OF STATEMENT

Date: 17th Sep 2025

Description of work

1) To unload 1 machine from loading bay, shift and position to 7th floor via cargo lift

2) To hoist up 1 unit bar feeder to 7th floor through shutter gate opening by using platform and mobile crane

Machine Spec

1 unit Ikura Bar Feeder

Dim: 400 x 60 x 110 cm

Wt: 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

/25

Time of Operation

09:45 am

Location of Operation

#### Blk 3 A.M.K Industrial Pk 2A #07-01

#### JP Precision Engineering

Equipment Use

1 x 100 ton mobile crane

1 x lorry crane

1 x platform

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew should be composed to the following but not limited by other trained personnel in respect of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Load up bar feeder onto platform

- Secured bar feeder onto platform with strapping belt

- Rigger to insert sling to crane hook

- Secured sling to rigging point of platform

- Mobile crane to hoist up platform together with bar feeder to 7th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers onto bar feeder stand

- Shift out bar feeder from platform

- Position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175