# METHOD OF STATEMENT

Date: 27th Mar 2025

Description of work

1) To hoist down 1 unit machine from 5th floor through swing gate opening by using platform and mobile/lorry crane, load onto lorry

2) To hoist up 1 unit new machine to 5th floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

Hoist down

1 unit LV600 CNC

Dim: 1.9 x 2.3 x 2.5 m / Wt: 4, 500 kg

Hoist up

1 unit PM1100 CNC

Dim: 2.9 x 2.3 x 2.5 m / Wt: 6, 400 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/04/25 (Sat)

Time of Operation

09:30 am

Location of Operation

#### Blk 3 A.M.K Pk 2A #05-08

#### Ace United Precision Engineering

Equipment Use

1 x 100 ton mobile crane

2-3 x 633 lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Riggers to attached sling to crane hook

- Secured sling to rigging point of platform

- Mobile crane to hoist up platform together with machine to 5th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert rollers under machine bed

- Use of manual pulley and slowly release to control moving speed of machine

- Shift out machine from platform

- Position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175