# METHOD OF STATEMENT

Date: 09th Sep 2024

Description of work

To hoist down machineries from 6th floor through wall hacking opening by using platform & mobile crane, load up onto lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/09/24 (Sat)

Time of Operation

10:00 am

Location of Operation

#### Blk 30 Kallang Place #06-16

#### Youprint Productions Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 30 ton mobile crane

3-4 x lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of 2 x mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones and lifting in operation signage

- 1 x 50ton mobile crane to hoist up platform to 6th floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x 30ton mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Use of manual pulley and slowly pull in machine into platform

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine from platform

- Swing crane and lower down machine to the ground

- Load up machine onto lorry chassis

- Secured with strapping belt

- Transport and deliver machine to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175