# METHOD OF STATEMENT

Date: 10th June 2024

Description of work

1) To hoist down 2 unit machine from 3rd floor through wall hacking opening by using platform, mobile/lorry crane and load onto lorry chassis

2) To shift out 5 unit small machine to loading bay via cargo lift, hoist and load up onto lorry chassis

Machine Spec

1 unit PM860 Machining Center

Dim: 300 x 220 x 235 cm / Wt: 6, 500 kg

1 unit ML480 CNC Lathe

Dim: 260 x 140 x 175 / Wt: 3, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/06/24 (subject to approval)

Time of Operation

09:45 am

Duration of Operation

Estimate: 09:45 am – 15:00 pm

Location of Operation

#### Blk 3027 Ubi Rd 1 #03-118

#### M & E Mould Engineering

Equipment Use

1 x 50 ton mobile crane

3-4 x lorry loader

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Barricade off work area with safety cones lifting in operation signage

- Mobile crane to hoist up platform to 3rd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production into platform

- Use of manual pulley and slowly pull in machine into platform

- Riggerman to insert sling to crane hook and secure to rigging point of machine

- Mobile crane to hoist up machine from platform

- Lower down machine to the ground

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175