# METHOD OF STATEMENT

Date: 18th July 2024

Description of work

1) To hoist down machineries from 6th floor through wall hacking opening by using platform & mobile crane, load up onto forwarder lorry chassis

2) To shift out small machine and accessories to loading bay via cargo lift, hoist and load up onto forwarder lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/08/24 (Fri)

Time of Operation

09:45 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1 #06-07

#### Moveon Technologies Pte Ltd

Equipment Use

1 x 100 ton mobile crane

1 x 50 ton mobile crane

2 x lorry loader

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 2 x mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones and lifting in operation signage

- 1 x 100ton mobile crane to hoist up platform to 6th floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x 50ton mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production into platform

- Use of manual pulley and slowly pull in machine into platform

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- 100ton mobile crane to hoist up machine from platform

- Lower down machine to the ground

- Load up machine onto forwarder lorry

- Repeat process till all machine have been hoist down

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175