# METHOD OF STATEMENT

Date: 31st Oct 2025

Description of work

1) To hoist down 1 unit machine from 2nd floor through swing gate opening by using platform, mobile/lorry crane and load onto lorry chassis

2) To shift out 2 units Grinder machine from 3rd floor to loading bay via cargo lift

Machine Spec

Via hoisting

1 unit Ewag Rotoline machine

Dim: 295 x 160 x 210 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/11/25

Time of Operation

10:00 am

Location of Operation

#### Blk 512 Chai Chee Lane #02-13

#### Stamford Precision Tools Pte Ltd

Equipment Use

2-3 x lorry crane

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Skill Mover

Job Scope

- Deployment of lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Lorry crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production into platform

- Riggerman to insert sling to crane hook and secure to rigging point of machine

- Lorry crane to hoist up machine from platform

- Swing crane and lower down machine to the ground

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to being transport out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175