# METHOD OF STATEMENT

Date: 14th Mar 2024

Description of work

To hoist up 1 unit forklift from ground floor to 2nd floor through shutter door opening by using platform and lorry crane

To hoist down 1 unit used forklift from 2nd floor through shutter door opening by using platform and lorry crane, load onto forwarder truck

Machine Spec

Dim: 280 x 100 x 200cm

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 60 Tuas Bay Drive

#### Sankyu (S) Pte Ltd

Equipment Use

2 x lorry loader

1 x platform

Wire/ Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Lorry crane to hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out forklift from production and load onto platform

- Attached sling to crane hook and secured onto rigging point of forklift.

Lorry crane to hoist down forklift and load onto lorry chassis

- Lorry crane to hoist up new forklift to 2nd floor

- Lower down forklift onto platform

- Use of manual pulley to slowly release and control moving speed of forklift

- Shift out forklift from platform

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175