# METHOD OF STATEMENT

Date: 26th Sep 2016

Description of work

To hoist up 1 unit Takisawa CNC Lathe NEX-110 machine to 2nd floor, shift and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

30/09/2016 – 09:30 am

Location of Operation

#### Blk 1 A.M.K Tech 1 #02-11

HY II Precision Engineering

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Pallet truck /rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any, will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc…) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd level opening

- Lower down machine slowly onto platform

- Insert pallet truck/rollers to below machine

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175