# METHOD OF STATEMENT

Date: 27th Sep 2017

Description of work

1) To shift out and hoist down 18 unit Bar Feeder and 10 Cases of long materials from 2nd floor, load onto lorry chassis and transport to new location

2) To shift out smaller machine from 2nd floor via cargo lift, load onto lorry at loading bay and transport to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

04-05/10/2017   –   09:30 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #02-03/05

Turntech Precision Engineering Pte Ltd

Equipment Use

3-4 x lorry crane

Wire/webbing sling

1 x platform

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Supervisor to ensure workers working near swing gate opening or on top platform to put on safety harness and attached it to life line provided

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine/bar feeder from production room to swing gate area

- Load machine onto platform

- Lorry crane to hoist down machine/bar feeder from 2nd floor

- Load machine/bar feeder onto lorry chassis

- Secured machine/bar feeder with strapping belt prior to moving out of premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175