# METHOD OF STATEMENT

Date: 26th Oct 2017

Description of work

To shift out wooden cases from ground floor, hoist up to 3rd floor via swing gate opening by using lorry crane

Case Spec

4 cases of 1 set Tubes

Dim: 417 x 84 x 44 cm

Wt: 772 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date/Time of Operation

01/11/2017   -   09:30 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #03-03

MMI Systems Pte Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Insert webbing sling through case base and hook onto crane hook

- Hoist up cases to 3rd floor via swing gate opening

- Insert pallet truck to case base

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175