# METHOD OF STATEMENT

Date: 05th Nov 2019

Description of work

To shift out and hoist down 1 unit Work Cell from 3rd floor by using platform and lorry crane, using forklift to load onto wooden base for packing

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

19/11/2019

Time Of Operation

09:00 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #03-03

MMI Systems Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire/webbing sling

1 x platform

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Barricade off work area

- Hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out equipment from production and load onto platform

- Secured machine onto platform with strapping belt

- Lorry crane to hoist down platform together with equipment to ground floor

- Using forklift to fork up Work Cell from platform and load onto wooden base

- Packing to form a case

- After packing, load onto forwarder lorry and transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175