# METHOD OF STATEMENT

Date: 02nd May 2017

Description of work

To shift out and hoist down 2 set of MDWA New Build from 3rd floor to ground floor by using platform and lorry crane

Load machine onto wooden base for packing, after packing load onto forwarder lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

05/05/2017   -   09:30 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #03-03

MMI Systems Pte Ltd

Equipment Use

1 x lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift and load machine into platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to ground floor

- Load machine onto wooden base for packing

- After packing load case onto forwarder lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be dispose and clear

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175