# METHOD OF STATEMENT

Date: 22nd May 2018

Description of work

To shift out and hoist down sets of MDWA New Build from 3rd floor to ground floor by using lorry crane, load onto wooden base for packing, after packing load onto forwarder lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

Month Of June 2018   -   Hoisting schedule for every Friday Except 15th and 29th)

1st , 8th and 22nd of June 2018

Time Of Operation

09:30 am

Location of Operation

#### Blk 10 Kaki Bukit Ave 1 #03-03

MMI Systems Pte Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out equipment from production room to swing gate area

- Insert webbing to equipment hoisting point

- Crane to hoist down equipment from 3rd floor to ground floor

- Load equipment onto wooden base for packing

- After packing load cases onto forwarder lorry and transport to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be dispose and clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175