# METHOD OF STATEMENT

Date: 04th July 2016

Description of work

1) To hoist down 1 unit Leadwell T7 CNC Lathe machine and 2 unit Moriseiki NV 5000 from

3rd floor and load onto lorry chassis

2) To hoist up 1 unit Eco Mill 800 CNC to 3rd floor, shift and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09/07/2016   -   09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 # 03-04

Unique Precision Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Wire/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton lorry crane to hoist up machine to 3rd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine base

- Push out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175