# METHOD OF STATEMENT

Date: 10th Sep 2018

Description of work

To hoist down 1 unit Drop Tester from 7th floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

15/09/2018   -   09:30 AM

Location of Operation

Blk 3015A Ubi Rd 1 #07-06

Rexsson Technology Pte Ltd)

Equipment Use

1 x 50 ton mobile crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who not involved enter unintentionally.

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade off operation area

- Hoist up platform to 7th floor swing gate opening

- Secure front left and right of platform onto building pillar using belt to prevent platform swing outward

- Shift machine into platform

- Secured machine onto platform using strapping belt

- Hoist down platform together with machine to ground floor and load onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175