# METHOD OF STATEMENT

Date: 16th Aug 2016

Description of work

To hoist up 1 unit Milltap 700 CNC Tapping machine to 4th floor by using platform and mobile/lorry crane

Working Area

Reserve car park lot no: 10-18

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24/08/2016   -   13:00 pm  – 15:00 pm

Location of Operation

#### Blk 3023 Ubi Rd 3 # 04-09

(Origin Precision Technology)

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger and signalman will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 4th level opening

- Tied back platform to the competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 4th floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel:62824175