# METHOD OF STATEMENT

Date: 22nd Apr 2022

Description of work

1) To hoist down 1 unit machine from 2nd floor, transport to front side of building and hoist up   to 2nd floor

2) To hoist up 1 unit new to 2nd floor through wall hacking opening by using platform and mobile/lorry crane

Machine Spec

Hoist down/up (From #02-66 to #02-70)

1 unit QT 6T

Dim: 170 x 130 x 170 cm

Wt: 1, 900 kg

Hoist up to #02-70

1 unit Mazak QTE200 SG

Dim: 190 x 166 x 170 cm

Wt: 3, 400 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate schedule:

15/05/22 – 31/05/22

Time of Operation

09:30 am

Location of Operation

#### Blk 31 Defu Lane 10 #02-66 & #02-70

Ching Yi Technology Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

2 x lorry

1 x scissor platform

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 2nd wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the wall hacking opening slab

- Tied back platform to a competent point and secured using 1 x lorry loader

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175