# METHOD OF STATEMENT

Date: 14th Sep 2021

Description of work

1) To hoist up 8 unit machine to 3rd floor by using platform and mobile/lorry crane

2) To hoist down 2 unit Feeler QM-32SA from 3rd floor and load onto buyer lorry

Machine Spec

Hoist up

6 unit Brother S700X1

Dim: 228 x 236 x 255 cm

Wt: 2, 242 kg each

2 unit Feeler CNC

Dim: 350 x 231 x 255 cm

Wt: 5, 470 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/09/21

Time of Operation

09:30 am

Location of Operation

#### Blk 31 Defu Lane 10 #03-66/68

Alcotec Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

2 x lorry

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Mobile crane to hoist up platform to 3rd wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the wall hacking opening slab

- Tied back platform to a competent point and secured using 1 x 45 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175