# METHOD OF STATEMENT

Date: 16th May 2018

Description of work

To hoist up 2 unit CNC Lathe machine to 2nd floor by using platform and mobile/lorry crane

Machine Spec

1 unit PM 1500 CNC

Dim: 2.3m x 3.6m x 2.7m

Wt: 8, 700 kgs

1 unit TP 750 CNC

Dim: 1.7m x 2.4m x2.6m

Wt: 4, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 16-30/06/2018   -   09:30 AM

Location of Operation

#### Blk 4010 Ang Mo Kio Ave 10 #02-05

Alma-Tech Precision Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x 45 ton lorry crane

1 x platform

Pallet truck /rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any, will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile/lorry crane to hoist up machine to 2nd level opening

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175