# METHOD OF STATEMENT

Date: 20 Nov 2014

Description of work

To unload 1 unit Oven from lorry chassis, shift and position into factory premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24/11/2014

Location of Operation

#### No 11 Tuas Ave 12

- Equipment Use

1 x lorry crane

Rollers / pallet truck

Wire/webbing sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane with load to park at designated hoisting area for the lift

- Barricade operation area

- Hoist down oven from lorry chassis

- Insert pallet truck/rollers to oven base

- Shift and position to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Kevin Wongg

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175