# METHOD OF STATEMENT

Date: 11 Apr 2014

Description of work

To hoist up 1 unit machine to 5th floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

17/04/2014 - 09:30 am

Location of Operation

#### No 6 Ang Mo Kio Electronics Park Road

- Equipment Use

50 ton mobile crane

1 x platform

Wire sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger man and signalman will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Lifting crew is pre-briefed prior to commencement of job                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade operation area

- Load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist up platform together with machine to 5th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Workers to insert rollers/pallet truck to below machine, shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be dispose and clear

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175