# METHOD OF STATEMENT

Date: 18 Sep 2014

Description of work

1) To hoist down 1 unit Global Advantage CMM machine from 3rd level to ground floor for packing. After packing, shift cases to car park shelter for storage

2) To shift and hoist down 1 unit Global Advantage CMM machine from 3rd level to 2nd level

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/09/2013 - 09:30 am – 17:00 pm

Location of Operation

#### No 6 Woodlands Loop

- Equipment Use

1 x 50 ton lorry crane

1 x 50 ton mobile crane

1 x platform

Heavy duty rollers

Wire sling

Shackle

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Lifting crew is pre-briefed prior to commencement of job                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc) Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd level opening

- 2-3 m tagline will be use to secure and control the swing motion of platform

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production area and load into platform

- 50 ton crane to hoist down machine

- Load machine onto wooden base for packing

- After packing, shift cases to car park shelter for storage

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175